Radio App Turn On Sequence and Functions…

(All app icons are in Radio Controller folder in Main Monitor Window)

Using Icom radio (with or without RSPdx)

- Turn on battery, power supply and IC-7100 in sequence

- If I want to use Win4Icom with Icom radio…

  - Open Win4Icom.

  - If I want to use Win4Icom Spectrum Scope, do NOT open Console.

  - Win4Icom Spectrum Scope will run off of RSPdx

- If I want to use SDR Console with Icom radio…

  - Open SDR Console first.

  - Open Win4Icom…

  - In SDR Console, check “External Radio” section for status:  Engage External Radio.  Only one Rig Controller and Panadapter can connect at a time.

  - Spectrum Scope will run off of RSPdx.

- If I want to use DX Labs with Icom radio… Only if SDR Console is not yet open. Otherwise a duplicate version will be opened via DX Labs.

  - Open DX Labs Launcher first. Click “Start”. The following will open in sequence:

  - SDR Console auto starts

  - Win4Icom will auto start X seconds after Console as indicated in DX Lab Launcher Config screen

  - In SDR Console:  Connect External Radio May auto track

  - DX Lab components will open as selected in Launcher

- Turn off all in reverse order

Using RSPdx

- Open SDR Console (or any other SDR app) with or without External Radio.